Beresford Network Design

Student no: 22075927

Student name: Siri Chandana Pothuri

SP22075927.pkt

Table of Contents

Introduction

Network Design Summary

Logical Diagram

Summary Addressing Tables

Network Design Details

Appendix

Introduction

This network design report presents the design and implementation of an enterprise scale network for Beresford. The report contains a summary of the network design spanning both sites. The network designed is a secure, scalable, and redundant architecture that supports communication, remote site connectivity, and effective routing across multiple VLANs as well as internet access.

The network spans two main geographic locations: Newcastle and Maitland. It supports a variety of departments such as Accounting, Marketing, Sales, Office, Engineering, and Management which are split into VLANs across both sites. The design ensures high availability through the implementation of Hot Standby Router Protocol (HSRP) at the Newcastle core, Spanning Tree, inter-VLAN routing using router-on-a-stick, a GRE tunnel, and OSPF for scalability and efficiency.

This report outlines the summary of the network design, the logical topology, VLAN segmentation and device addressing, along with the details of the network’s design to achieve a reliable network that meets the organisation’s technical and functional requirements.

Network Design Summary

Core sites and structure

Newcastle serves as the primary data center with redundant routers (Newc1 and Newc2) running HSRP. Maitland is a branch office connected to Newcastle via a GRE tunnel, which runs OSPF through area 0. Internet access is simulated through a router, ISP, connected to the Newcastle routers serially configured with multiple loopbacks and point-to-point links for external connectivity testing.

Network Addressing

The private network address allocated for Beresford, 192.168.0.0/19, is grouped into 7 VLANs using VLSM to account for all of Beresford’s needs, as well as an extra set of subnets to account for links, tunnels and OSPF addressing requirements which are to come from the internal addressing. The subnets contain enough host space for the required size for Beresford’s departments as well as extra for future growth. Newcastle hosts the VLANs Accounting, Marketing, Sales, Office and Management, which is also configured as the native VLAN to handle traffic through the network. Maitland hosts the Mait Office and Management VLANs.

Switching

The switches all have rapid-PVST+ enabled with STP balancing VLANs between the Newcastle switches in order to provide load balancing and redundancy. SwM and SwB are connected via EtherChannel to aggregate bandwidth and ensure failover. HSRP is implemented on Newc1 and Newc2 for all VLANs except the ones hosted by Maitland, and the default gateway for hosts is the HSRP virtual IP address. To load balance routing, Newc1 is configured as the active router for most VLANs with priority 110, while Newc2 serves as standby (priority 100).

Network Routing

Routing is implemented via router-on-a-stick at Maitland and Newcastle with the VLANs they host, while HSRP is enabled on Newcastle’s routers. OSPF is enabled on the network, with traffic being forwarded through the connection between Newcastle and Maitland sites., labelled area 0. Newcastle’s routes are configured in area 1, and Maitland’s in area 2, which allows for an effective grouping of OSPF routing.

Security

Security is managed on Beresford’s network by implementing ACLs where needed to separate VLAN traffic and restrict management access to important devices.

WAN

PAT is configured to allow internet connectivity through network address translation on Newc1 and Newc2 with the public address allocation specified by Beresford.

Logic Diagram

Note on change from proposed topology: SwM’s use of Fa0/5 to connect to PCM has been changed to Fa0/18 to keep VLAN port addressing consistent across switches

Summary Addressing Table/s

Network addressing scheme as described in Beresford’s technical specifications

| Usage | IP Block | Assigned Device |
| --- | --- | --- |
| Private block for Beresford | 192.168.0.0/19 | Internal |
| Public block for Newc1/2 | 200.50.0.0/28 | Across Newc 1 and 2 |
| ISP P2P link (Newc1) | 100.1.1.0/30 | Newc1 - ISP Router |
| ISP P2P link (Newc2) | 100.1.2.0/30 | Newc2 - ISP Router |
| ISP Loopback 1 | 33.33.33.33/32 | ISP Router |
| ISP Loopback 2 | 44.44.44.44/32 | ISP Router |

The IPv4 subnetting table for Beresford’s sites and VLANs is on the next page, containing requirements and solutions.

| VLAN and Number | Subnet address | Subnet Mask | Hosts (with growth) | Required Size/Maximum hosts in subnet | Usable IP range | Default gateway |
| --- | --- | --- | --- | --- | --- | --- |
| Accounting (10) | 192.168.2.0/25 | 255.255.255.128 | 50 (100) | 126 | 192.168.2.1 - 192.168.2.126 | 192.168.2.1 |
| Marketing (20) | 192.168.3.0/24 | 255.255.255.0 | 80 (160) | 254 | 192.168.3.1 – 192.168.3.254 | 192.168.3.1 |
| Sales (30) | 192.168.4.0/24 | 255.255.255.0 | 90 (180) | 254 | 192.168.4.1 – 192.168.4.254 | 192.168.4.1 |
| Office (40) | 192.168.0.0/23 | 255.255.254.0 | 200 (400) | 510 | 192.168.0.1 – 192.168.1.254 | 192.168.0.1 |
| Management (VLAN 150, including Native) for Newc and Mait | 192.168.5.0/28 192.168.5.16/28 | 255.255.255.240 | 10 | 14 | 192.168.5.1 – 192.168.5.14 192.168.5.17 – 192.168.5.30 | 192.168.5.1 192.168.5.17 |
| Mait Office (50) | 192.168.2.128/25 | 255.255.255.128 | 40 (80) | 126 | 192.168.2.129 – 192.168.2.254 | 192.168.2.129 |
| Mait Engineering (60) | 192.168.5.64/26 | 255.255.255.192 | 30 (60) | 62 | 192.168.5.65 – 192.168.5.126 | 192.168.5.65 |
| Blackhole (VLAN 600) |  |  | All unused ports |  |  |  |
| Links and tunnels | 192.168.6.0/30 192.168.6.4/30 192.168.6.8/30 192.168.6.12/30 | 255.255.255.252 |  | 2 | 192.168.6.1 – 192.168.6.2 192.168.6.5- 192.168.6.6 192.168.6.9 – 192.168.6.10 192.168.6.13 – 192.168.6.14 |  |
| OSPF router addressing | 192.168.6.16/30 192.168.6.20/30 | 255.255.255.252 |  | 2 | 192.168.6.17 – 192.168.6.18 192.168.6.21 – 192.168.6.22 |  |

Network Design Details

| Device | Interface | IP address | Subnet Mask | Description | Notes |
| --- | --- | --- | --- | --- | --- |
| ISP | S0/1/0 | 100.1.1.1 | 255.255.255.252 | Newc 1 |  |
|  | S0/1/1 | 100.1.2.1/30 | 255.255.255.252 | Newc2 |  |
|  | ISP loopback 1 | 33.33.33.33 | 255.255.255.255 | Internet |  |
|  | ISP loopback 2 | 44.44.44.44 | 255.255.255.255 | Internet |  |
| Newc1 | S0/1/0 | 100.1.1.2 | 255.255.255.252 | ISP | G0/0/0.10, G0/0/0.30, G0/0/0.150 active for HSRP |
|  | S0/1/1 | 192.168.6.13 | 255.255.255.252 | Mait tunnel carrier |  |
|  | Tunnel0 | 192.168.6.5 | 255.255.255.252 | GRE tunnel |  |
|  | G0/0/0.10 | 192.168.2.2 | 255.255.255.128 | Accounting |  |
|  | G0/0/0.20 | 192.168.3.2 | 255.255.255.0 | Marketing |  |
|  | G0/0/0.30 | 192.168.4.2 | 255.255.255.0 | Sales |  |
|  | G0/0/0.40 | 192.168.0.2 | 255.255.254.0 | Office |  |
|  | G0/0/0.150 | 192.168.5.2 | 255.255.255.240 | Management |  |
|  | G0/0/1 | 192.168.6.1 | 255.255.255.252 | Newc2 |  |
| Newc2 | S0/1/0 | 100.1.2.2 | 255.255.255.252 | ISP | G0/0/0.20, G0/0/0.40 active for HSRP G0/0/0 is connected to SWT |
|  | G0/0/0.10 | 192.168.2.3 | 255.255.255.128 | Accounting |  |
|  | G0/0/0.20 | 192.168.3.3 | 255.255.255.0 | Marketing |  |
|  | G0/0/0.30 | 192.168.4.3 | 255.255.255.0 | Sales |  |
|  | G0/0/0.40 | 192.168.0.3 | 255.255.254.0 | Office |  |
|  | G0/0/0.150 | 192.168.5.3 | 255.255.255.240 | Management |  |
|  | G0/0/1 | 192.168.6.2 | 255.255.255.252 | Newc1 |  |
| Newc3 | S0/1/0 | 192.168.6.9 | 255.255.255.252 | Mait |  |
|  | G0/0/0 | 192.168.5.7 | 255.255.255.240 | SWB |  |
| Mait | S0/1/0 | 192.168.6.10 | 255.255.255.252 | Newc3 | G0/0/0 is connected to PSW |
|  | S0/1/1 | 192.168.6.14 | 255.255.255.252 | Newc1 tunnel carrier |  |
|  | Tunnel0 | 192.168.6.6 | 255.255.255.252 | GRE tunnel |  |
|  | G0/0/0.50 | 192.168.2.129 | 255.255.255.128 | Mait office |  |
|  | G0/0/0.60 | 192.168.5.65 | 255.255.255.192 | Mait Engineering |  |
|  | G0/0/0.150 | 192.168.5.17 | 255.255.255.240 | Management Mait |  |
| SwT | Vlan150 | 192.168.5.4 | 255.255.255.240 | Switch | VLAN 150 on STP priority |
|  | Fa0/1 |  |  | Newc1 |  |
|  | Fa0/2 |  |  | Newc2 |  |
|  | Fa0/3-4 |  |  | SwM trunk |  |
|  | Fa0/5 |  |  | SwB link |  |
|  | Fa0/6-8 |  |  | Accounting |  |
|  | Fa0/9-11 |  |  | Marketing |  |
|  | Fa0/12-14 |  |  | Sales |  |
|  | Fa0/15-17 |  |  | Office |  |
|  | Fa0/18-20 |  |  | Management |  |
| SwM | Vlan150 | 192.168.5.5 | 255.255.255.240 | Switch | VLAN 10 and 20 on STP priority |
|  | Fa0/1-2 |  |  | SwB trunk (etherchannel) |  |
|  | Fa0/3-4 |  |  | SwT trunk |  |
|  | Fa0/18 |  |  | PCM in Management |  |
|  | Fa0/6-8 |  |  | Accounting |  |
|  | Fa0/9-11 |  |  | Marketing |  |
|  | Fa0/12-14 |  |  | Sales |  |
|  | Fa0/15-17 |  |  | Office |  |
|  | Fa0/18-20 |  |  | Management |  |
| SwB | Vlan 150 | 192.168.5.6 | 255.255.255.240 | Switch | VLAN 30 and 40 on STP priority |
|  | Fa0/1-2 |  |  | Trunk to SwM (Etherchannel) |  |
|  | Fa0/3 |  |  | Link to SwT |  |
|  | Fa0/4 |  |  | Newc3 |  |
|  | Fa0/11 |  |  | PCS in Sales |  |
|  | Fa0/5-7 |  |  | Accounting |  |
|  | Fa0/8-10 |  |  | Marketing |  |
|  | Fa0/11-13 |  |  | Sales |  |
|  | Fa0/14-16 |  |  | Office |  |
|  | Fa0/17-19 |  |  | Management |  |
| PSW | Vlan150 | 192.168.5.18 | 255.255.255.240 | Switch |  |
|  | Fa0/1 |  |  | Link to Mait |  |
|  | Fa0/5 |  |  | PCE in Engineering |  |
|  | Fa0/2-4 |  |  | Mait Office |  |
|  | Fa0/5-7 |  |  | Engineering |  |
|  | Fa0/8-10 |  |  | Management |  |
| PCE | Fa/0 | DHCP from Engineering VLAN (192.168.5.66 – 192.168.5.126) | 255.255.255.192 | PC to PSW |  |
| PCM | Fa/0 | 192.168.5.14 | 255.255.255.240 | PC to SwM | Last address of management vlan |
| PCS |  | DHCP from Sales VLAN (192.168.4.2-192.168.4.254) | 255.255.255.0 | PC to SwB |  |

Appendix

Device configs

ISP:

Current configuration : 1236 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname ISP

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

no ip cef

no ipv6 cef

!

username ISP password 7 0822455D0A16

!

no ip domain-lookup

!

spanning-tree mode pvst

!

interface Loopback1

ip address 33.33.33.33 255.255.255.255

!

interface Loopback2

ip address 44.44.44.44 255.255.255.255

!

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 100.1.1.1 255.255.255.252

encapsulation ppp

clock rate 128000

!

interface Serial0/1/1

ip address 100.1.2.1 255.255.255.252

encapsulation ppp

clock rate 128000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 200.50.0.0 255.255.255.240 100.1.1.2

ip route 200.50.0.0 255.255.255.240 100.1.2.2

!

ip flow-export version 9

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

!

end

Newc1

Current configuration : 3503 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname Newc1

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

ip dhcp excluded-address 192.168.4.1

ip dhcp excluded-address 192.168.4.255

ip dhcp excluded-address 192.168.4.2

ip dhcp excluded-address 192.168.4.3

!

ip dhcp pool SALES

network 192.168.4.0 255.255.255.0

default-router 192.168.4.2

domain-name cisco.com

!

no ip cef

no ipv6 cef

!

username Newc1 password 7 0822455D0A16

!

no ip domain-lookup

ip domain-name cisco.com

!

spanning-tree mode pvst

!

interface Tunnel0

ip address 192.168.6.5 255.255.255.252

mtu 1476

tunnel source Serial0/1/1

tunnel destination 192.168.6.14

!

!

interface GigabitEthernet0/0/0

no ip address

ip nat inside

duplex auto

speed auto

!

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.2.2 255.255.255.128

ip ospf 1 area 1

standby 10 ip 192.168.2.1

standby 10 priority 110

standby 10 preempt

!

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.3.2 255.255.255.0

ip ospf 1 area 1

standby 20 ip 192.168.3.1

standby 20 preempt

!

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.4.2 255.255.255.0

ip ospf 1 area 1

ip access-group SALES_RESTRICT in

standby 30 ip 192.168.4.1

standby 30 priority 110

standby 30 preempt

!

interface GigabitEthernet0/0/0.40

encapsulation dot1Q 40

ip address 192.168.0.2 255.255.254.0

ip ospf 1 area 1

standby 40 ip 192.168.0.1

standby 40 preempt

!

interface GigabitEthernet0/0/0.150

encapsulation dot1Q 150 native

ip address 192.168.5.2 255.255.255.240

ip ospf 1 area 1

ip nat inside

standby 150 ip 192.168.5.1

standby 150 priority 110

standby 150 preempt

!

interface GigabitEthernet0/0/1

ip address 192.168.6.1 255.255.255.252

duplex auto

speed auto

!

interface Serial0/1/0

ip address 100.1.1.2 255.255.255.252

encapsulation ppp

ip nat outside

!

interface Serial0/1/1

ip address 192.168.6.13 255.255.255.252

ip ospf 1 area 0

ip nat inside

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 192.168.6.17

log-adjacency-changes

area 1 range 192.168.0.0 255.255.254.0

area 1 range 192.168.2.0 255.255.255.128

area 1 range 192.168.3.0 255.255.255.0

area 1 range 192.168.4.0 255.255.255.0

area 1 range 192.168.5.0 255.255.255.240

passive-interface default

no passive-interface Serial0/1/1

network 192.168.6.4 0.0.0.3 area 0

default-information originate

!

ip nat pool NATPOOL1 200.50.0.0 200.50.0.15 netmask 255.255.255.240

ip nat inside source list 1 pool NATPOOL1 overload

ip classless

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/1 200

ip route 192.168.6.14 255.255.255.255 Serial0/1/1

!

ip flow-export version 9

!

!

access-list 1 permit 192.168.0.0 0.0.7.255

ip access-list extended SALES_RESTRICT

permit ip 192.168.4.0 0.0.0.255 192.168.4.0 0.0.0.255

deny ip 192.168.4.0 0.0.0.255 any

permit udp any any eq bootps

permit udp any any eq bootpc

ip access-list extended MANAGEMENT_ACCESS

permit tcp host 192.168.5.14 any eq 22

permit tcp host 192.168.5.14 any eq telnet

deny tcp any any eq telnet

deny tcp any any eq 22

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class MANAGEMENT_ACCESS in

password 7 0822455D0A16

login local

transport input ssh

!

End

Newc2

Current configuration : 2832 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname Newc2

!

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

username Newc2 password 7 0822455D0A16

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0/0.10

encapsulation dot1Q 10

ip address 192.168.2.3 255.255.255.128

ip ospf 1 area 1

standby 10 ip 192.168.2.1

standby 10 preempt

!

interface GigabitEthernet0/0/0.20

encapsulation dot1Q 20

ip address 192.168.3.3 255.255.255.0

ip ospf 1 area 1

standby 20 ip 192.168.3.1

standby 20 priority 110

standby 20 preempt

!

interface GigabitEthernet0/0/0.30

encapsulation dot1Q 30

ip address 192.168.4.3 255.255.255.0

ip ospf 1 area 1

ip access-group SALES_RESTRICT in

standby 30 ip 192.168.4.1

standby 30 preempt

!

interface GigabitEthernet0/0/0.40

encapsulation dot1Q 40

ip address 192.168.0.3 255.255.254.0

ip ospf 1 area 1

standby 40 ip 192.168.0.1

standby 40 priority 110

standby 40 preempt

!

interface GigabitEthernet0/0/0.150

encapsulation dot1Q 150 native

ip address 192.168.5.3 255.255.255.240

ip ospf 1 area 1

ip nat inside

standby 150 ip 192.168.5.1

standby 150 preempt

!

interface GigabitEthernet0/0/1

ip address 192.168.6.2 255.255.255.252

duplex auto

speed auto

!

interface Serial0/1/0

ip address 100.1.2.2 255.255.255.252

encapsulation ppp

ip nat outside

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 192.168.6.18

log-adjacency-changes

area 1 range 192.168.0.0 255.255.254.0

area 1 range 192.168.2.0 255.255.255.128

area 1 range 192.168.3.0 255.255.255.0

area 1 range 192.168.4.0 255.255.255.0

area 1 range 192.168.5.0 255.255.255.240

default-information originate

!

ip nat pool NATPOOL1 200.50.0.0 200.50.0.15 netmask 255.255.255.240

ip nat inside source list 1 pool NATPOOL1 overload

ip classless

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/0/0 200

!

ip flow-export version 9

!

!

access-list 1 permit 192.168.0.0 0.0.7.255

ip access-list extended SALES_RESTRICT

permit ip 192.168.4.0 0.0.0.255 192.168.4.0 0.0.0.255

deny ip 192.168.4.0 0.0.0.255 any

ip access-list extended MANAGEMENT_ACCESS

permit tcp host 192.168.5.14 any eq 22

permit tcp host 192.168.5.14 any eq telnet

deny tcp any any eq 22

deny tcp any any eq telnet

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

access-class MANAGEMENT_ACCESS in

password 7 0822455D0A16

login

transport input ssh

!

!

!

end

SWT

Current configuration : 3249 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname SWT

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

username SWT privilege 1 password 7 0822455D0A16

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 150 priority 4096

!

interface FastEthernet0/1

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/10

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/11

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/13

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/14

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/15

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/16

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/17

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/18

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/19

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/20

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/21

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/1

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/2

switchport access vlan 600

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan150

ip address 192.168.5.4 255.255.255.240

!

ip default-gateway 192.168.5.1

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

line vty 5 15

password 7 0822455D0A16

login local

transport input ssh

!

!

!

!

end

SWM

Current configuration : 3305 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname SWM

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

username SWM privilege 1 password 7 0822455D0A16

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 10,20 priority 4096

!

interface Port-channel1

description EtherChannel to SwB

switchport trunk native vlan 150

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/5

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/10

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/11

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/13

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/14

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/15

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/16

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/17

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/18

switchport access vlan 150

!

interface FastEthernet0/19

switchport access vlan 150

!

interface FastEthernet0/20

switchport access vlan 150

!

interface FastEthernet0/21

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/1

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/2

switchport access vlan 600

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan150

ip address 192.168.5.5 255.255.255.240

!

ip default-gateway 192.168.5.1

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

line vty 5 15

password 7 0822455D0A16

login local

transport input ssh

!

!

!

!

End

SWB

Current configuration : 3379 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname SWB

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

username SWB privilege 1 password 7 0822455D0A16

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 30,40 priority 4096

!

interface Port-channel1

description EtherChannel to SwM

switchport trunk native vlan 150

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/2

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/3

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 150

switchport trunk allowed vlan 10,20,30,40,50,60,150

switchport mode trunk

!

interface FastEthernet0/5

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/7

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/8

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/10

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/11

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/12

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/13

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/14

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/15

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/16

switchport access vlan 40

switchport mode access

!

interface FastEthernet0/17

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/18

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/19

switchport access vlan 150

switchport mode access

!

interface FastEthernet0/20

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/21

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/1

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/2

switchport access vlan 600

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan150

ip address 192.168.5.6 255.255.255.240

!

ip default-gateway 192.168.5.1

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

line vty 5 15

password 7 0822455D0A16

login local

transport input ssh

!

!

!

!

End

Newc3

Current configuration : 1128 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname Newc3

!

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

username Newc3 password 7 0822455D0A16

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 192.168.5.7 255.255.255.240

ip ospf 1 area 1

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 192.168.6.9 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

!

ip classless

ip route 192.168.5.2 255.255.255.255 GigabitEthernet0/0/0

!

ip flow-export version 9

!

!

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

!

!

!

end

Mait

Current configuration : 2344 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname Mait

!

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

ip dhcp excluded-address 192.168.5.65

ip dhcp excluded-address 192.168.5.127

!

ip dhcp pool ENGINEERING

network 192.168.5.64 255.255.255.192

default-router 192.168.5.65

domain-name cisco.com

!

!

!

ip cef

no ipv6 cef

!

!

!

username Mait password 7 0822455D0A16

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface Tunnel0

ip address 192.168.6.6 255.255.255.252

mtu 1476

tunnel source Serial0/1/1

tunnel destination 192.168.6.13

!

!

interface GigabitEthernet0/0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0/0.50

encapsulation dot1Q 50

ip address 192.168.2.129 255.255.255.128

ip ospf 1 area 2

!

interface GigabitEthernet0/0/0.60

encapsulation dot1Q 60

ip address 192.168.5.65 255.255.255.192

ip ospf 1 area 2

ip access-group ENGINEERING_RESTRICT in

!

interface GigabitEthernet0/0/0.150

encapsulation dot1Q 150 native

ip address 192.168.5.17 255.255.255.240

ip ospf 1 area 2

!

interface GigabitEthernet0/0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

ip address 192.168.6.10 255.255.255.252

!

interface Serial0/1/1

ip address 192.168.6.14 255.255.255.252

ip ospf cost 2000

ip ospf 1 area 0

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 192.168.6.21

log-adjacency-changes

area 2 range 192.168.2.128 255.255.255.128

area 2 range 192.168.5.64 255.255.255.192

area 2 range 192.168.5.16 255.255.255.240

passive-interface default

no passive-interface Serial0/1/1

network 192.168.6.4 0.0.0.3 area 0

!

ip classless

ip route 192.168.6.13 255.255.255.255 Serial0/1/1

!

ip flow-export version 9

!

!

ip access-list extended ENGINEERING_RESTRICT

permit ip 192.168.5.64 0.0.0.63 192.168.5.64 0.0.0.63

deny ip 192.168.5.64 0.0.0.63 192.168.0.0 0.0.255.255

permit ip 192.168.5.64 0.0.0.63 any

permit udp any any eq bootps

permit udp any any eq bootpc

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

!

!

!

End

PSW

Current configuration : 2956 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname PSW

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

no ip domain-lookup

ip domain-name cisco.com

!

username PSW privilege 1 password 7 0822455D0A16

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport trunk native vlan 150

switchport trunk allowed vlan 50,60,150

switchport mode trunk

!

interface FastEthernet0/2

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/3

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/4

switchport access vlan 50

switchport mode access

!

interface FastEthernet0/5

switchport access vlan 60

switchport mode access

!

interface FastEthernet0/6

switchport access vlan 60

!

interface FastEthernet0/7

switchport access vlan 60

!

interface FastEthernet0/8

switchport access vlan 150

!

interface FastEthernet0/9

switchport access vlan 150

!

interface FastEthernet0/10

switchport access vlan 150

!

interface FastEthernet0/11

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/12

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/13

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/14

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/15

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/16

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/17

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/18

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/19

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/20

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/21

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/22

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/23

switchport access vlan 600

switchport mode access

shutdown

!

interface FastEthernet0/24

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/1

switchport access vlan 600

switchport mode access

shutdown

!

interface GigabitEthernet0/2

switchport access vlan 600

switchport mode access

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan150

ip address 192.168.5.18 255.255.255.240

!

ip default-gateway 192.168.5.17

!

banner motd ^C UNAUTHORISED ACCESS PROHIBITED ^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

login local

transport input ssh

line vty 5 15

password 7 0822455D0A16

login local

transport input ssh

!

!

!

!

end

ACL configs

Newc1 and Newc2

Standard IP access list 1 – For NAT access

10 permit 192.168.0.0 0.0.7.255

Extended IP access list SALES_RESTRICT – For Sales VLAN (G0/0/0.30)

Allows intra-vlan access:

10 permit ip 192.168.4.0 0.0.0.255 192.168.4.0 0.0.0.255

Denies access to other VLANs:

20 deny ip 192.168.4.0 0.0.0.255 any

Allows DHCP access:

30 permit udp any any eq bootps

40 permit udp any any eq bootpc

Extended IP access list MANAGEMENT_ACCESS – For Management VLAN (G0/0/0.150)

Allows PCM, SSH and Telnet access to all devices:

10 permit tcp host 192.168.5.14 any eq 22

20 permit tcp host 192.168.5.14 any eq telnet

Denies any other devices from accessing SSH and Telnet

30 deny tcp any any eq telnet

40 deny tcp any any eq 22

Mait

Extended IP access list ENGINEERING_RESTRICT – For Engineering VLAN (G0/0/0.60)

Allows Intra-vlan access:

10 permit ip 192.168.5.64 0.0.0.63 192.168.5.64 0.0.0.63

Denies access to other vlans

20 deny ip 192.168.5.64 0.0.0.63 192.168.0.0 0.0.255.255

Allows internet access:

30 permit ip 192.168.5.64 0.0.0.63 any

Allows DHCP access:

40 permit udp any any eq bootps

50 permit udp any any eq bootpc

NAT config

On Newc1 and Newc2:

access-list 1 permit 192.168.0.0 0.0.7.255

ip nat pool NATPOOL1 200.50.0.0 200.50.0.15 netmask 255.255.255.240

ip nat inside source list 1 pool NATPOOL1 overload

From show ip nat statistics command:

access-list 1 pool NATPOOL1

pool NATPOOL1: netmask 255.255.255.240

start 200.50.0.0 end 200.50.0.15

The nat inside interfaces on Newc1 and Newc2 are G0/0/0.150 (and S0/1/1 on Newc1 for NAT for Mait), and the outside interfaces are S0/1/0 which connects to the ISP.

DHCP config

For the sales DHCP, on Newc1:

ip dhcp excluded-address 192.168.4.1

ip dhcp excluded-address 192.168.4.255

ip dhcp excluded-address 192.168.4.2

ip dhcp excluded-address 192.168.4.3

ip dhcp pool SALES

network 192.168.4.0 255.255.255.0

default-router 192.168.4.2

domain-name cisco.com

The excluded addresses belong to the subnet’s default gateway, the g0/0/0.30 addresses for Newc1 and Newc2, and the broadcast address. The default router is the default gateway and the virtual HSRP address for the Sales VLAN.

For the Engineering DHCP, on Mait

ip dhcp excluded-address 192.168.5.65

ip dhcp excluded-address 192.168.5.127

ip dhcp pool ENGINEERING

network 192.168.5.64 255.255.255.192

default-router 192.168.5.65

domain-name cisco.com

The excluded addresses belong to the subnet’s default gateway which is the g0/0/0.60 address on Mait, and the broadcast address. The default router is the default gateway for the Engineering VLAN